This requires ISCE2 with stack processor. I always follow this guide for installation of isce, mintpy https://github.com/yunjunz/conda-envs

DEM

$ sardem --bbox  -107 38 -104 40  --data COP -isce

% NOTE for this UAVSAR processing, we wont actually use the DEM here, but you need it to run prepareUAVSAR_coregStack.py. We will use the DEM provided with the UAVSAR data.

stripmapStack

- Download SLC Stack files (.slc, .ann, and .dop) from https://uavsar.jpl.nasa.gov.

- copy the wget commands from the UAVSAR into a terminal

- Put everything except for the .dop file inside a directory named “download”

- run:

$ prepareUAVSAR_coregStack.py -i download -o SLC -d swatch_00540_01_BC.dop -s 1

i = directory where data are downloaded, o = SLC folder, d = .dop file. Note that if there are multiple segments need to add command “- s 2” or “-s 3” . Segment 1 is default.

- create a “merged” directory and then create a soft link inside it to the SLC directory (inside the “merged” directory run:  ln -s ../SLC )

- run:

$ stackStripMap.py -s SLC -d /u/sar-r1/handwerg/insar/Colorado/AspenArea/DEM/DEM_GLO30/elevation.dem -a 8 -r 3 -W interferogram --nofocus -S uavsar_stack -t 1100 -u snaphu --filter_strength 0.4

  - set azimuth and range looks, and make sure that the time in days is enough large to cover all the time for a dense stack

  - This step creates the run files you need. You only will run a single run file to create the interferograms

- create interferograms:

$ source run_08_igram

(make sure to run it inside a screen, could take a while)

- Use jupyter notebooks to create the geometry files. Start with 1_prep_uavsar_geometry.  This notebook will take the geometry files from UAVSAR, then re-interpolate them to 1x1 looks so that you can then make them for the specific number of looks used in your processing.

- Use juypter notebook 2_prep_isce_MintPy_UAVSAR.ipynb  to create mintpy files for UAVSAR(if this is preferred). This is then used to geocode.

- Geocode the data using notebook 3_geocode.ipynb

Currently there is only a single pair of interferograms so no need for time series. Simply run the data into mintpy for formatting purposes and geocoding.

Calculate spatial coherence.

temporal_average.py ifgramStack.h5 -d coherence -o avgSpatialCoh.h5

Pixel tracking

python denseoffset_adj.py -m ../SLC/20230725/20230725.slc -s ../SLC/20230925/20230925.slc

window size and steps are hard coded into the denseOffset script

geocode using 4_geocode_offsets.ipynb

** you will need to make another set of the geometry files using the run_mulitlook_geometry.sh where the looks = skips.